# Product Requirements Specification (PRS)

Robot Control System
Version 1.0

Contents

1. Project Overview	3

2. Stakeholders	3

3. Goals and Success Criteria	4

3.1 Product Goals	4

3.2 Success Criteria	4

4. Scope Definition	5

4.1 In-Scope	5

4.2 Out-of-Scope (Current Version)	5

5. Functional Requirements	6

5.1 Live Telemetry Feed	6

5.2 Drive Controls	6

5.3 Movement Tracker	7

5.4 Position and Kinematics Display	7

5.5 Activity Log	8

5.6 System Status and Software Updates	8

5.7 System Actions API	9

6. Non-Functional Requirements	10

6.1 Reliability	10

6.2 Performance	10

6.3 Usability	10

6.4 Compatibility	10

7. API Contract	11

7.1 GET /get_live_data	11

7.2 POST /send_command	11

8. UI Structure	12

Robot Control	12

System Status	12

9. Visual Requirements	13

10. Acceptance Criteria (Formalized)	13

11. Risks and Mitigations	14

12. Roadmap (Post-Version-1)	14

# 1. Project Overview

The system shall provide a local web-based dashboard for real-time teleoperation and monitoring of a simulated mobile robot. The robot system shall publish operational state data to a local JSON file (mqtt_live_data.json) and expose HTTP endpoints to accept teleoperation and system commands.

The system shall consist of:

- A Python-based backend simulation service

- A browser-based, single-page HTML/JavaScript frontend

- File-based data exchange and REST communication

The system shall enable operators to:

- Monitor robot kinematics, health, and software state

- Issue movement and system-level commands

- Visualize robot trajectories graphically

# 2. Stakeholders

| Role | Responsibility |
| --- | --- |
| Product Owner | Defines functionality and product direction |
| Developers | Implement frontend and backend software |
| QA / Test | Verify correctness, stability, and usability |
| Operators | Use the system for robot control and monitoring |

# 3. Goals and Success Criteria

## 3.1 Product Goals

The system shall:

- Provide a reliable local control interface for a simulated robot

- Display real-time position, orientation, and kinematic metrics

- Enable remote teleoperation simulation

- Report system diagnostics and update status

- Provide traceable system and command activity logs

## 3.2 Success Criteria

The system shall be considered successful if:

- Real-time position and orientation are visualized correctly on the tracker graph

- Commands (forward/back/left/right/rotate) execute within one polling cycle

- Robot path history is accurately logged and displayed

- System updates and diagnostic information are visible in the UI

- The application runs without browser console errors due to missing DOM elements

- Reset position functionality clears backend and frontend state consistently

# 4. Scope Definition

## 4.1 In-Scope

The system shall include:

- A single-page UI (interface.html)

- HTML5 canvas-based movement tracker

- Teleoperation command interface

- Activity logging panel

- System diagnostic and update simulation

- Python backend:

  - mqtt_server.py

  - robot_system.py

  - run_system.py

- HTTP interfaces:

  - /get_live_data

  - /send_command

  - /system_action

## 4.2 Out-of-Scope (Current Version)

- Integration with physical robots or real hardware

- User authentication and authorization

- Persistent storage (beyond JSON)

- Packaging, installers, or production deployment workflows

# 5. Functional Requirements

## 5.1 Live Telemetry Feed

FR-01: The backend shall periodically write robot state data to mqtt_live_data.json.
FR-02: The data file shall contain the following data blocks:

- kinematics.current_position

- kinematics.statistics

- health

- drive_response

FR-03: The frontend shall poll /get_live_data at an interval between 300ms and 1000ms.
FR-04: The frontend shall update all UI elements upon successful data retrieval.

## 5.2 Drive Controls

FR-05: The UI shall provide controls for:

- Forward

- Backward

- Left

- Right

- Rotate Left

- Rotate Right

FR-06: The UI shall provide a speed slider adjustable from 10% to 100%.

FR-07: The frontend shall send commands using POST /send_command in JSON format:

{

"direction": "forward|backward|left|right|rotate_left|rotate_right",

"speed": 0.1–1.0

}

FR-08: The backend shall enqueue and execute commands sequentially.
FR-09: The backend shall write command execution results into drive_response.

## 5.3 Movement Tracker

FR-10: The UI shall display:

- Robot path (polyline)

- Start position indicator

- Current position indicator

- Orientation arrow

FR-11: The tracker shall support:

- Clear Path

- Center View

- Zoom In / Zoom Out

- Toggle Grid

- Reset Position

FR-12: Rotation-only commands shall modify orientation without modifying X/Y coordinates.

FR-13: Reset Position shall:

- Reset the robot pose to (0,0,0)

- Clear all stored path points

## 5.4 Position and Kinematics Display

FR-14: The system shall display:

- X, Y, Theta

- Instant velocity

- Total distance traveled

- Path points

- Average speed

- Movement efficiency

FR-15: All values shall update in real time from backend-provided telemetry.

## 5.5 Activity Log

FR-16: The system shall maintain a rolling activity log.
FR-17: The log shall include:

- Command submissions

- System events

- Position updates

FR-18: The log shall retain a configurable maximum number of entries.

## 5.6 System Status and Software Updates

FR-19: The UI shall display:

- Health overview

- Uptime

- CPU and memory usage

- Voltage

- Error count

FR-20: The UI shall display a Software Updates card with:

- Last check timestamp

- Total components

- Available updates

FR-21: "Check for Updates" shall trigger a backend simulation and update timestamps.

## 5.7 System Actions API

FR-22: The backend shall support the following system actions:

- diagnostics

- reboot

- factory_reset

- backup

- restore

- update_software

- reset_position

FR-23: The Reset Position action shall reset pose and clear path history.

# 6. Non-Functional Requirements

## 6.1 Reliability

NFR-01: The UI shall validate element existence before DOM manipulation.

## 6.2 Performance

NFR-02: Default polling interval shall be 500 ms.
NFR-03: Backend update loop shall operate at ≤10 Hz.
NFR-04: The system shall avoid excessive file I/O operations.

## 6.3 Usability

NFR-05: The UI shall render in a single-column layout at screen widths below 768px.

## 6.4 Compatibility

NFR-06: The UI shall be compatible with:

- Chrome

- Edge

- Firefox

# 7. API Contract

## 7.1 GET /get_live_data

Response:

{

"kinematics": {

"current_position": {"x":0,"y":0,"theta":0},

"statistics": {

"total_distance_traveled_m":0,

"path_points_logged":0,

"average_velocity_ms":0,

"movement_efficiency":0

}

},

"health": {},

"drive_response": {}

}

## 7.2 POST /send_command

{

"direction": "forward|rotate_left",

"speed": 0.5

}

Response:

{"status":"SUCCESS|ERROR","message":"..."}

# 8. UI Structure

## Robot Control

- Connection banner

- Position & Kinematics

- Drive Controls

- Activity Log

- Movement Tracker

## System Status

- Software Updates

- Diagnostics

- Logs and Actions

# 9. Visual Requirements

VR-01: Theme shall be dark mode only.
VR-02: Background: #000000
VR-03: Cards: #2b2b2b
VR-04: Graph: #1f1f1f
VR-05: Status indicators may use color semantics.

# 10. Acceptance Criteria (Formalized)

AC-01 Startup

- Position shows (0,0,0)

- UI loads without errors

AC-02 Command Execution

- Motion command changes position

- Rotation changes orientation

AC-03 Reset

- Position resets to origin

- Canvas clears

AC-04 Software Updates

- Timestamp updates

- UI renders labels in white

AC-05 Stability

- No null DOM write errors

# 11. Risks and Mitigations

| Risk | Mitigation |
| --- | --- |
| JSON race condition | Atomic file write |
| UI crash | DOM guards, try/catch |
| Backend failure | Health checks |
| Browser variance | Multi-browser testing |

# 12. Roadmap (Post-Version-1)

- WebSockets

- Authentication / RBAC

- Persistent storage

- Backend testing framework

- Exportable logs